Fused Deposition Modeling (FDM)

Developed in the late 80s.

Process Principle

Principle of FDM process and extruder

Filament is made of thermoplastic materials (Acrylonitrile butadiene styrene (ABS), Polylactide (PLA), other new materials).

Dual extruder machines exist; Temporary support structures can be made from water-soluble material, two colors.

Support Material           Modeling Material

https://www.youtube.com/watch?v=ik39_sv-wgQ

Commercial Systems

•Stratasys

–Dimension family

•Z resolution: 0.18 mm

•Build size: 8 x 8 x 12 inches

•Limited color

•Limited material types

•MakerBot

•Delta 3D Printer

•Ultimaker

•Cube from 3D Systems

•And many others

Stereolithography (SLA)

Process Principle

•SLA uses liquid photo-reactive resin

•Laser beam traces one layer on the surface of the resin

•Laser light cures and solidifies parts it hits

•The platform descends by one layer

•Support structure

–thin support lattice can be broken off

http://www.youtube.com/watch?feature=player_embedded&v=5L5vdpklrtU

History

-Developed in the 80s,  founded 3D Systems in 1986.

Commercial

•Two main families

–ProJet

–iPro

•Build volume: varies, can be very large

•Resolution up to 0.05mm

•Materials (only one can be used):

–photopolymers

–clear, opaque, temperature resistant, ceramic-like, abs-like

•Formlabs

–Smaller build volume

–Similar resolution

–Much less expensive

Selective Laser Sintering (SLS)/Direct Metal LaserSintering (DMLS)

Principle

•SLS and DMLS use a bed of small particles (made of plastic, metal, ceramic, or glass)

•High-power laser traces one layer on the surface of the powder bed fusing the particles

•The platform descends by one layer and more material is added

Features

•Laser and scanner system (similar do SLA but laser is more powerful)

•Bulk material can be preheated  (reduces the required energy to melt it)

•Materials (One material at a time, glass, polymers (e.g., nylon, polysterine), metals (e.g., steel, titanium, alloys), ceramic)

•Does not require support structure  (overhangs are supported by powder material)

Single- and Two-Component Powders

Raw Ni-alloy-Cu powder mixture               Raw Fe-Cu powder mixture (Tolochko et al. 2003)

Fe powder after sintering

Fe-Cu powder mixture after sintering

https://www.youtube.com/watch?v=BZLGLzyMKn4

Commercial Systems

•3D Systems (sPro family & Pro DM)

•EOS GmbH  (Formiga and EOSINT family)

•Requires powerful laser (30W for SLS, 400W for DMLS)

•Layer thickness: 0.02 – 0.08 mm

Sample Fabricated Parts

3D printed, titanium central wing spar                       Airbus wing bracket